DDR device internal clk is a ratio 16:1 of the IO clk

For controller clk 2133MHz, bandwidth because of the double edge is 4266Mbps, device internal clk is 266MHz.

How frequent of DDR refresh is independent of the IO clk, but a factor of RAM cell capacitator discharge

Latency:

L3: ~20ns

LLC: ~40 t0 50 ns

DRAM hit: ~100ns

DRAM miss: > 100ns

Inline ECC:

Internal clock 200MHZ

- Every time a reference is made to a new DRAM page, the full 8 KiB is transferred from the DRAM array to the DRAM sense amps. This uses a fair amount of power, and it makes sense to try to read the entire 8 KiB while it is in the sense amps. Although it is beyond the scope of today’s discussion, reading data from “open pages” uses only about 1/4 to 1/5 of the power required to read the same amount of data in “closed page” mode, where only one cache line is retrieved from each DRAM page.

- Data in the sense amps can be accessed at significantly lower latency — this is called “open page” access (or a “page hit”), and the latency is referred to as the “CAS latency”. On most systems the CAS latency is between 12.5 ns and 15 ns, independent of the frequency of the DRAM bus. If the data is not in the sense amps (a “page miss”), loading data from the array to the sense amps takes an additional latency of 12.5 to 15 ns. If the sense amps were holding other valid data, it would be necessary to write that data back to the array, taking an additional 12.5 to 15 ns. If the DRAM latency is not completely overlapped with the cache coherence latency, these increases will reduce the sustainable bandwidth according to Little’s Law: Bandwidth = Concurrency / Latency

From <https://sites.utexas.edu/jdm4372/tag/dram/>